ARTICLE XVIII

# Commercial Development Design Guidelines

§ 270-133.

I.

# § 270-134. Design standards and guidelines for commercial establishments  in commercial zoning districts: Town Center Overlay, Freemans Bridge Rd Corridor, and Highway Commercial Zoning Districts.

- Purpose and objectives.

  - The following guidelines and standards are intended to  encourage building designs and site layouts for commercial and mixed-use developments that contribute to the Town of Glenville as a unique place by reflecting its physical character and adding to it in complementary ways.

  - The Town of Glenville would like to further the sense of place for the established Town Center area, the emerging

Freemans Bridge Road Corridor area, and promote the rural agricultural character of the Route 5 area as well as provide opportunities for commercial and mixed-use developments in line with the scale and character of each area described below:

    - The Town Center area as defined by the Town Center Overlay District promotes a traditional architectural and visual environment that incorporates the elements of institutional, commercial, vehicular and pedestrian environments into an integrated commercial and civic design, as stated in the purpose statement in § 270-133A and objectives in § 270-133B.

    - The Freemans Bridge Road corridor as defined by the Freemans Bridge Road District  is  a  destination  gateway to the Glenville for economic development in a size and scale that are harmonious with the surrounding area and complementary to a bicycle and pedestrian-friendly environment, as stated in the purpose statement in this Subsection A and objectives in Subsection B.

    - The Route 5 area is a mixed-use corridor promoting highway commercial uses which encourages a hybrid of commercial and light industrial uses to attract development best suited for rural highway corridors, as stated in  §  270-19.1A;  and  rural  residential  and agricultural uses which look to maintain low-density residential and agricultural-type uses consistent with § 270-14A.

  - These guidelines and standards have been developed to promote a basic level of architectural variety, compatible scale, pedestrian access and use of open space which is reflective of the goals and objectives of the Town Comprehensive Plan. They are by no means intended to limit creativity; they are intended to augment the requirements of site plan review of Article XVI with additional criteria that are specific to commercial developments.

- Guidelines and standards.

  - Guidelines and standards. The guidelines and standards below are intended to be used as a basis for design by developers proposing commercial projects. They are also intended as an evaluation tool by the Planning and Zoning Commission in its site plan review process. These guidelines apply to all commercial projects and mixed use developments. Guidelines are  not  mandatory  but  explain  the  design  objectives. Standards  are  mandatory  unless  their  strict application will result in undue hardship as determined by the Planning and Zoning Commission.

  - Modifications and  waivers.  The  Planning  and  Zoning Commission may  waive  one  or  more  of  the  specific requirements of this article upon a showing by the applicant that the regulation imposes an undue hardship due to such factors as  existing  conditions,  site  topography  or  site configuration. The Planning and Zoning Commission shall approve the  minimum  waiver necessary  to  allow  the application to be approved. The applicant for any such waiver shall have the burden of showing that the proposed project with such waiver will have a minimum negative effect on aesthetics and compatibility with neighborhood character.

- Colors and materials.

  - Colors utilized for building materials shall be compatible with the surrounding area. Examples of incompatible colors in the

Freemans Bridge Road Corridor, the Route 5 corridor, and the Town Center include metallic, neon, fluorescent and/or primary colors.

  - Materials. Traditional materials (masonry, bricks, wood and stone) are  preferred;  however, contemporary  materials (glass, steel, concrete and plastic/vinyl siding) may be considered if they are treated in a manner complementary to the area. Examples of incompatible materials include exposed concrete block, metal siding and reflective glass.

    - Facade colors shall be low-reflectance, subtle, neutral or earth-tone colors.  The  use  of  high-intensity  colors, metallic colors, black or fluorescent colors is prohibited.

    - Exposed neon tubing is not an acceptable feature for building trim or accent areas.

    - Predominant exterior building materials shall not include the following:  smooth-faced  concrete  block,  tilt-up  concrete panels, or prefabricated steel panels.

- Architectural features and details.

  - Exterior facades, including eaves, pilasters, cornices, windows (and window surrounds), door balusters, canopies, fascias and roofs, shall be proportionate with the building. The scale of new construction, including the arrangement of

windows, doors and other openings within the building  facade, shall be compatible with the surrounding area.

  - To the extent practicable, accessory structures shall conform to the finishes and colors established for the principal building.

  - Blank walls shall not be permitted along any exterior walls facing a street, parking lot, or pedestrian area. Walls without windows must have architectural features similar to the front facade including materials, colors, and details.

  - There must be breaks in a linear row of buildings at a minimum of every 50 feet incorporating varied rooflines, building heights, and other architectural features as described in Subsection D(1).

  - Building orientation and entrances. The front facade of buildings should be oriented towards the principal frontage with an entrance in this facade. Primary entrances should be accentuated and  prominently  visible  with  architectural details distinguishing said entrance. Architectural details for primary entrances include: a recessed entrance, chamfered corner, awning, entranceway  roof, sidelight, transom, or  adjacent windows  to  the  doorway, and/or  additional mouldings with expression lines.

    - Secondary entrances are permitted on secondary frontages or  on  the  corner  for  corner  properties.  Secondary entrances shall be included in the building design for buildings consisting of 20,000 square feet or more of gross floor area on secondary frontages.

  - Window standards. Smoked, reflective, or black glass in windows is prohibited on the ground floor. For commercial and mixed-use developments, ground-floor windows shall be a maximum of 12 inches to 20 inches above the ground measured from the base of the building beneath the window sill to the window glass.

- Roofs.

  - Roofs shall be in keeping with the character of adjacent buildings. Variations in rooflines should be used to add interest to smaller buildings and reduce the massive scale of large buildings.

  - Roofs shall have the following features:

- For flat roofs, parapets shall conceal rooftop equipment such as HVAC units from public view.

- For sloping roofs that do not exceed the average height of the supporting walls, an average slope greater than or equal to one foot of the vertical rise for every three feet of horizontal run and less than or equal to one foot of vertical rise for every one foot of horizontal run.

- Overhanging eaves, extending no less than two feet past the supporting walls.

- Loading and service entrances and screening.

  - Loading and service entrances and outdoor storage should be designed to minimize visual and noise impacts on surrounding neighborhoods. These areas, when visible from adjoining properties and/or public streets, must be screened, recessed or enclosed. Appropriate locations for loading and outdoor storage areas include areas between buildings where more than one building is located on a site and such buildings are not more than 40 feet apart, or on those sides of buildings that do not have customer entrances.

  - Areas of outdoor storage, trash collection or compaction, loading docks, service entrances, overhead doors, or other such uses shall not be located between the building and any public street or within 20 feet of any public sidewalk, on-site

roadway or internal pedestrian way. These provisions shall not apply  to  individual  outdoor  trash  receptacles  and recyclable materials  receptacles  intended  for  use  by  pedestrians or visitors.

  - All exterior mechanical,  electrical,  communication  and service equipment, which includes all ground, wall, and roof- mounted equipment, shall be screened with landscaping and/ or appropriate  architectural  treatments  such  as  parapets, walls, or  fences  consistent  with  the  materials  listed  in Subsection C(2) or via landscaping.

  - All rooftop mechanical equipment must be concealed or integrated into the roof form or screened from view at ground level. Dumpsters, except for temporary construction dumpsters, must be screened from public view by a solid fence or row of evergreen shrubs.

  - Unroofed areas for the storage and sale of seasonal inventory shall be permanently defined and screened with walls and/or fences that screen such areas from public view. Materials,  colors, and design of screening walls,  fences  or  covers  shall be compatible with those used as predominant materials and colors on the building. If such areas are to be covered, then   the covering shall conform to those used as predominant materials and colors on the building.

- Drive-through windows and canopies.

- Drive-through windows. Drive-through windows should be located to  the  side  or  rear  of  the  building.  Pedestrian connectivity should be provided in accordance with  Subsection K. Drive-through window  design and materials should be consistent with the overall building style, form, and materials.

- Canopies for drive-through operations and walk-up windows.

  - Drive-through and walk-up canopies should be integrated into the overall building design and designed as an extension of the building.

    - Drive-through window design and materials should be consistent with the overall building style, form, and materials.

    - Roofline elements and columns should be coordinated with the overall design of the building.

    - Drive-through windows shall not be located along the principal frontage. In the case of a corner lot with two frontages, the Planning and Zoning Commission shall determine the  appropriate  location  for  a  drive- through window.

    - Pedestrian walkways must have clear visibility and be enhanced with  pavings  or  markings  when  they intersect with drive-through aisles.

    - Walk-up windows and take-out windows can be used for ice cream shops especially in conjunction with an outdoor dining area or seating area.

    - Drive-through locations can also have walk-up windows for pedestrians and bicyclists.

    - Lighting must be shielded or recessed into the canopy or roofline  in  the  drive-through  window  area  to prevent glare.

    - Menu boards, loudspeakers, stacking spaces, drive- through windows,  service  and  loading  shall  be located only in interior side or rear yards.

    - Loudspeakers, automobile service order devices, and similar instruments shall be located at least 20 feet from any street right-of-way and 20 feet from any property boundaries.

    - A drive-through menu board shall comply with the standards for freestanding or monument signs.

    - All	service	and	mechanical	equipment	shall	be screened from adjacent sites and public rights-of-way.

    - Trash receptacles must be provided and screened from adjacent sites and public rights-of-way.

- Lighting.

  - Architectural/pedestrian-scaled lighting should enhance the area without introducing glare on, off or above the site.

  - Architectural/pedestrian-scaled lighting shall consist of the Sternberg Vintage Lighting Park Ridge series fixture with a fourteen-foot standard black paint Lincoln pole. Lightbulbs shall be LEDs with a maximum color temperature less than 4,000 Kelvin; 3,000 Kelvin and under is preferred. Shields and night sky optics shall also be used on light fixtures to project light  down  onto  the  parking  lot,  sidewalk,  or landscaped area. While Sternberg is the preferred manufacturer, the Town may accept an equivalent light  fixture manufacturer/supplier, but all the above specifications relative to style, height, color, bulb, shielding, and maximum wattage must be met, and shall be consistent with existing Sternberg fixtures already in use in Glenville for parking lots and pedestrian areas. Off-street parking lot lighting shall be a

maximum	of	18	feet	in	height	with	the	above	design requirements.

  - Electric service transmission lines and connections should be subsurface to  reduce  the  amount  of  utility  poles  and associated wires.

I. Amenities. Amenities shall be required and included on the landscape plan to be reviewed by the Planning and Zoning Commission as part of site plan review. The plan shall include, but not be limited to, benches, bike racks, outdoor dining areas, transit shelters,  screened  trash  receptacles  and  recyclable materials receptacles. These amenities are to be illustrated on the landowner's portion of the site plan and will be privately owned and maintained.

- Parking lot orientation and parking lot landscaping.

  - Parking should be located to the side and rear of the buildings in the Town Center and Freemans Bridge Road Corridor and/ or to the rear of the lot.

  - Landscaping shall be used on the perimeter of all parking lots with 10 or more spaces in an effort to soften the visual impact of parking lots from the street and neighboring land uses, particularly from  residential  uses.  Interior  landscaped parking islands and peninsulas are allowed, but where they are used, the preferred layout is that of larger islands and peninsulas instead of numerous small islands. Each development site and parking lot is unique, with proposed landscaping to be evaluated and determined by the Planning and Zoning Commission on a case-by-case basis.

  - Landscaping techniques  using  green  infrastructure  best management practices is encouraged including rain gardens, vegetated swales, filter strips, and stormwater planters.  Further details about site infiltration practices can be found in the Capital District Regional Planning Commission Green Infrastructure Toolkit found at cdrpc.org.

- Pedestrian connectivity.

  - Sidewalks are to be constructed and located pursuant to Chapter 221, Sidewalks. A minimum of six feet in width shall be provided.

  - Continuous internal pedestrian walkways, no less than six feet in width, shall be provided from the public sidewalk or right-of-way to  the  principal  customer  entrance  of  all principal buildings on the site. At a minimum, walkways shall connect focal points of pedestrian activity such as, but not limited to, transit stops, street crossings, outdoor benches and seating areas, and building entry points, and shall feature adjoining landscaped areas no less than five feet in width that include trees, grass, shrubs, benches, flower beds, ground covers, or other such materials for no less than 50% of their length. These areas must comply with the minimum planting area requirements set forth in § 270-141E of this chapter.

  - Internal pedestrian walkways no less than six feet in width shall be provided along any facade featuring a customer entrance, and any facade abutting public parking areas. Such walkways shall be located at least six feet from the facade of the building  to  provide  planting  beds  for  foundation landscaping, except where features such as arcades or entry ways are part of the facade. These areas must comply with

the minimum  planting  area  requirements  set  forth  in  § 270-141E of this chapter.

  - Internal pedestrian walkways provided in conformance with Subsection K(2)  or  (3)  above  should  provide  weather protection features  such  as  awnings  or  arcades  at  all customer entrances.

  - All internal pedestrian walkways shall be distinguished from driving surfaces through the use of durable, low-maintenance surface materials such as pavers, bricks, scored concrete or other materials acceptable to the Planning and Zoning Commission to enhance pedestrian safety and comfort, as well as the attractiveness of the walkways.

  - Internal pedestrian walkways shall be maintained by the property owner.

- Central features and community spaces.

  - Large commercial establishments shall contribute to the establishment or enhancement  of community and public spaces by providing at least two of the following: patio/ seating area, pedestrian plaza with benches, transportation center, window shopping walkway, outdoor playground area, kiosk area, water feature, clock tower, or and/or a focal feature or amenity that, in the judgment of the Planning and

Zoning Commission, adequately enhances such community and public spaces.

  - Bike racks and trash receptacles. Each  large  commercial development site plan shall include bike racks and trash receptacles. Trash  receptacles  shall  be  Victor  Stanley Concourse series FC-12 or equal. These amenities are to be provided on the site plan and will be privately owned and maintained.

  - Any such areas or amenities constructed or included pursuant to Subsection I(1) or (2) above, shall have direct access to the public sidewalk  network,  and  such  features  shall  be constructed of  materials  that  are  compatible  with  the principal materials of the building and landscape.

- Gateways.

  - The Town of Glenville would like to create gateways at the entry areas to the Town to enhance the sense of identity. The appearance and prominence of Town gateways shall be improved through the use of appropriate signage, landscaping, by more uniform setbacks, building design, public art, and amenities. The seven gateways are: Saratoga Road (Route 50), Freemans Bridge Road (Route 911F), Amsterdam Road (Route 5), Sacandaga Road (Route 147), Glenridge Road (Route 914V), the Exit 26 bridge, and the Route 50/Village of Scotia municipal boundary.

  - Any commercial establishment adjacent to, or within  1,000  feet of, a Town gateway shall contribute  to  enhancement  of the gateway through additional landscaping, as  required  by the Planning and Zoning Commission. Examples of potential contributions	include	perennial	low-maintenance groundcover, use of native trees and native plantings.

- Application procedures.  The  application  procedures  to  be followed are the same as for those required of site plan review applications as detailed in § 270-108 of the Zoning Code.

# §

# § 270-136.

- Purpose and objectives.

  - The following guidelines and standards are intended to  encourage building designs and site layouts for commercial and mixed-use developments that contribute to the Town of Glenville as a unique place by reflecting its physical character and adding to it in complementary ways.

  - The Town of Glenville would like to further the sense of place for the established Town Center area, the emerging

Freemans Bridge Road Corridor area, and promote the rural agricultural character of the Route 5 area as well as provide opportunities for commercial and mixed-use developments in line with the scale and character of each area described below:

    - The Town Center area as defined by the Town Center Overlay District promotes a traditional architectural and visual environment that incorporates the elements of institutional, commercial, vehicular and pedestrian environments into an integrated commercial and civic design, as stated in the purpose statement in § 270-133A and objectives in § 270-133B.

    - The Freemans Bridge Road corridor as defined by the Freemans Bridge Road District  is  a  destination  gateway to the Glenville for economic development in a size and scale that are harmonious with the surrounding area and complementary to a bicycle and pedestrian-friendly environment, as stated in the purpose statement in this Subsection A and objectives in Subsection B.

    - The Route 5 area is a mixed-use corridor promoting highway commercial uses which encourages a hybrid of commercial and light industrial uses to attract development best suited for rural highway corridors, as stated in  §  270-19.1A;  and  rural  residential  and agricultural uses which look to maintain low-density residential and agricultural-type uses consistent with § 270-14A.

  - These guidelines and standards have been developed to promote a basic level of architectural variety, compatible scale, pedestrian access and use of open space which is reflective of the goals and objectives of the Town Comprehensive Plan. They are by no means intended to limit creativity; they are intended to augment the requirements of site plan review of Article XVI with additional criteria that are specific to commercial developments.

- Guidelines and standards.

  - Guidelines and standards. The guidelines and standards below are intended to be used as a basis for design by developers proposing commercial projects. They are also intended as an evaluation tool by the Planning and Zoning Commission in its site plan review process. These guidelines apply to all commercial projects and mixed use developments. Guidelines are  not  mandatory  but  explain  the  design  objectives. Standards  are  mandatory  unless  their  strict application will result in undue hardship as determined by the Planning and Zoning Commission.

  - Modifications and  waivers.  The  Planning  and  Zoning Commission may  waive  one  or  more  of  the  specific requirements of this article upon a showing by the applicant that the regulation imposes an undue hardship due to such factors as  existing  conditions,  site  topography  or  site configuration. The Planning and Zoning Commission shall approve the  minimum  waiver necessary  to  allow  the application to be approved. The applicant for any such waiver shall have the burden of showing that the proposed project with such waiver will have a minimum negative effect on aesthetics and compatibility with neighborhood character.

- Colors and materials.

- Colors utilized for building materials shall be compatible with the surrounding area. Examples of incompatible colors in the

- Freemans Bridge Road Corridor, the Route 5 corridor, and the Town Center include metallic, neon, fluorescent and/or primary colors.

- Materials. Traditional materials (masonry, bricks, wood and stone) are  preferred;  however, contemporary  materials (glass, steel, concrete and plastic/vinyl siding) may be considered if they are treated in a manner complementary to the area. Examples of incompatible materials include exposed concrete block, metal siding and reflective glass.

- Facade colors shall be low-reflectance, subtle, neutral or earth-tone colors.  The  use  of  high-intensity  colors, metallic colors, black or fluorescent colors is prohibited.

- Exposed neon tubing is not an acceptable feature for building trim or accent areas.

- Predominant exterior building materials shall not include the following:  smooth-faced  concrete  block,  tilt-up  concrete panels, or prefabricated steel panels.

- Architectural features and details.

- Exterior facades, including eaves, pilasters, cornices, windows (and window surrounds), door balusters, canopies, fascias and roofs, shall be proportionate with the building. The scale of new construction, including the arrangement of

- windows, doors and other openings within the building  facade, shall be compatible with the surrounding area.

- To the extent practicable, accessory structures shall conform to the finishes and colors established for the principal building.

- Blank walls shall not be permitted along any exterior walls facing a street, parking lot, or pedestrian area. Walls without windows must have architectural features similar to the front facade including materials, colors, and details.

- There must be breaks in a linear row of buildings at a minimum of every 50 feet incorporating varied rooflines, building heights, and other architectural features as described in Subsection D(1).

- Building orientation and entrances. The front facade of buildings should be oriented towards the principal frontage with an entrance in this facade. Primary entrances should be accentuated and  prominently  visible  with  architectural details distinguishing said entrance. Architectural details for primary entrances include: a recessed entrance, chamfered corner, awning, entranceway  roof, sidelight, transom, or  adjacent windows  to  the  doorway, and/or  additional mouldings with expression lines.

- Secondary entrances are permitted on secondary frontages or  on  the  corner  for  corner  properties.  Secondary entrances shall be included in the building design for buildings consisting of 20,000 square feet or more of gross floor area on secondary frontages.

- Window standards. Smoked, reflective, or black glass in windows is prohibited on the ground floor. For commercial and mixed-use developments, ground-floor windows shall be a maximum of 12 inches to 20 inches above the ground measured from the base of the building beneath the window sill to the window glass.

- Roofs.

- Roofs shall be in keeping with the character of adjacent buildings. Variations in rooflines should be used to add interest to smaller buildings and reduce the massive scale of large buildings.

- Roofs shall have the following features:

- For flat roofs, parapets shall conceal rooftop equipment such as HVAC units from public view.

- For sloping roofs that do not exceed the average height of the supporting walls, an average slope greater than or equal to one foot of the vertical rise for every three feet of horizontal run and less than or equal to one foot of vertical rise for every one foot of horizontal run.

- Overhanging eaves, extending no less than two feet past the supporting walls.

- Loading and service entrances and screening.

- Loading and service entrances and outdoor storage should be designed to minimize visual and noise impacts on surrounding neighborhoods. These areas, when visible from adjoining properties and/or public streets, must be screened, recessed or enclosed. Appropriate locations for loading and outdoor storage areas include areas between buildings where more than one building is located on a site and such buildings are not more than 40 feet apart, or on those sides of buildings that do not have customer entrances.

- Areas of outdoor storage, trash collection or compaction, loading docks, service entrances, overhead doors, or other such uses shall not be located between the building and any public street or within 20 feet of any public sidewalk, on-site

- roadway or internal pedestrian way. These provisions shall not apply  to  individual  outdoor  trash  receptacles  and recyclable materials  receptacles  intended  for  use  by  pedestrians or visitors.

- All exterior mechanical,  electrical,  communication  and service equipment, which includes all ground, wall, and roof- mounted equipment, shall be screened with landscaping and/ or appropriate  architectural  treatments  such  as  parapets, walls, or  fences  consistent  with  the  materials  listed  in Subsection C(2) or via landscaping.

- All rooftop mechanical equipment must be concealed or integrated into the roof form or screened from view at ground level. Dumpsters, except for temporary construction dumpsters, must be screened from public view by a solid fence or row of evergreen shrubs.

- Unroofed areas for the storage and sale of seasonal inventory shall be permanently defined and screened with walls and/or fences that screen such areas from public view. Materials,  colors, and design of screening walls,  fences  or  covers  shall be compatible with those used as predominant materials and colors on the building. If such areas are to be covered, then   the covering shall conform to those used as predominant materials and colors on the building.

- Drive-through windows and canopies.

- Drive-through windows. Drive-through windows should be located to  the  side  or  rear  of  the  building.  Pedestrian connectivity should be provided in accordance with  Subsection K. Drive-through window  design and materials should be consistent with the overall building style, form, and materials.

- Canopies for drive-through operations and walk-up windows.

- Drive-through and walk-up canopies should be integrated into the overall building design and designed as an extension of the building.

- Drive-through window design and materials should be consistent with the overall building style, form, and materials.

- Roofline elements and columns should be coordinated with the overall design of the building.

- Drive-through windows shall not be located along the principal frontage. In the case of a corner lot with two frontages, the Planning and Zoning Commission shall determine the  appropriate  location  for  a  drive- through window.

- Pedestrian walkways must have clear visibility and be enhanced with  pavings  or  markings  when  they intersect with drive-through aisles.

- Walk-up windows and take-out windows can be used for ice cream shops especially in conjunction with an outdoor dining area or seating area.

- Drive-through locations can also have walk-up windows for pedestrians and bicyclists.

- Lighting must be shielded or recessed into the canopy or roofline  in  the  drive-through  window  area  to prevent glare.

- Menu boards, loudspeakers, stacking spaces, drive- through windows,  service  and  loading  shall  be located only in interior side or rear yards.

- Loudspeakers, automobile service order devices, and similar instruments shall be located at least 20 feet from any street right-of-way and 20 feet from any property boundaries.

- A drive-through menu board shall comply with the standards for freestanding or monument signs.

- All	service	and	mechanical	equipment	shall	be screened from adjacent sites and public rights-of-way.

- Trash receptacles must be provided and screened from adjacent sites and public rights-of-way.

- Lighting.

- Architectural/pedestrian-scaled lighting should enhance the area without introducing glare on, off or above the site.

- Architectural/pedestrian-scaled lighting shall consist of the Sternberg Vintage Lighting Park Ridge series fixture with a fourteen-foot standard black paint Lincoln pole. Lightbulbs shall be LEDs with a maximum color temperature less than 4,000 Kelvin; 3,000 Kelvin and under is preferred. Shields and night sky optics shall also be used on light fixtures to project light  down  onto  the  parking  lot,  sidewalk,  or landscaped area. While Sternberg is the preferred manufacturer, the Town may accept an equivalent light  fixture manufacturer/supplier, but all the above specifications relative to style, height, color, bulb, shielding, and maximum wattage must be met, and shall be consistent with existing Sternberg fixtures already in use in Glenville for parking lots and pedestrian areas. Off-street parking lot lighting shall be a

- maximum	of	18	feet	in	height	with	the	above	design requirements.

- Electric service transmission lines and connections should be subsurface to  reduce  the  amount  of  utility  poles  and associated wires.

- I. Amenities. Amenities shall be required and included on the landscape plan to be reviewed by the Planning and Zoning Commission as part of site plan review. The plan shall include, but not be limited to, benches, bike racks, outdoor dining areas, transit shelters,  screened  trash  receptacles  and  recyclable materials receptacles. These amenities are to be illustrated on the landowner's portion of the site plan and will be privately owned and maintained.

- Parking lot orientation and parking lot landscaping.

- Parking should be located to the side and rear of the buildings in the Town Center and Freemans Bridge Road Corridor and/ or to the rear of the lot.

- Landscaping shall be used on the perimeter of all parking lots with 10 or more spaces in an effort to soften the visual impact of parking lots from the street and neighboring land uses, particularly from  residential  uses.  Interior  landscaped parking islands and peninsulas are allowed, but where they are used, the preferred layout is that of larger islands and peninsulas instead of numerous small islands. Each development site and parking lot is unique, with proposed landscaping to be evaluated and determined by the Planning and Zoning Commission on a case-by-case basis.

- Landscaping techniques  using  green  infrastructure  best management practices is encouraged including rain gardens, vegetated swales, filter strips, and stormwater planters.  Further details about site infiltration practices can be found in the Capital District Regional Planning Commission Green Infrastructure Toolkit found at cdrpc.org.

- Pedestrian connectivity.

- Sidewalks are to be constructed and located pursuant to Chapter 221, Sidewalks. A minimum of six feet in width shall be provided.

- Continuous internal pedestrian walkways, no less than six feet in width, shall be provided from the public sidewalk or right-of-way to  the  principal  customer  entrance  of  all principal buildings on the site. At a minimum, walkways shall connect focal points of pedestrian activity such as, but not limited to, transit stops, street crossings, outdoor benches and seating areas, and building entry points, and shall feature adjoining landscaped areas no less than five feet in width that include trees, grass, shrubs, benches, flower beds, ground covers, or other such materials for no less than 50% of their length. These areas must comply with the minimum planting area requirements set forth in § 270-141E of this chapter.

- Internal pedestrian walkways no less than six feet in width shall be provided along any facade featuring a customer entrance, and any facade abutting public parking areas. Such walkways shall be located at least six feet from the facade of the building  to  provide  planting  beds  for  foundation landscaping, except where features such as arcades or entry ways are part of the facade. These areas must comply with

- the minimum  planting  area  requirements  set  forth  in  § 270-141E of this chapter.

- Internal pedestrian walkways provided in conformance with Subsection K(2)  or  (3)  above  should  provide  weather protection features  such  as  awnings  or  arcades  at  all customer entrances.

- All internal pedestrian walkways shall be distinguished from driving surfaces through the use of durable, low-maintenance surface materials such as pavers, bricks, scored concrete or other materials acceptable to the Planning and Zoning Commission to enhance pedestrian safety and comfort, as well as the attractiveness of the walkways.

- Internal pedestrian walkways shall be maintained by the property owner.

- Central features and community spaces.

- Large commercial establishments shall contribute to the establishment or enhancement  of community and public spaces by providing at least two of the following: patio/ seating area, pedestrian plaza with benches, transportation center, window shopping walkway, outdoor playground area, kiosk area, water feature, clock tower, or and/or a focal feature or amenity that, in the judgment of the Planning and

- Zoning Commission, adequately enhances such community and public spaces.

- Bike racks and trash receptacles. Each  large  commercial development site plan shall include bike racks and trash receptacles. Trash  receptacles  shall  be  Victor  Stanley Concourse series FC-12 or equal. These amenities are to be provided on the site plan and will be privately owned and maintained.

- Any such areas or amenities constructed or included pursuant to Subsection I(1) or (2) above, shall have direct access to the public sidewalk  network,  and  such  features  shall  be constructed of  materials  that  are  compatible  with  the principal materials of the building and landscape.

- Gateways.

- The Town of Glenville would like to create gateways at the entry areas to the Town to enhance the sense of identity. The appearance and prominence of Town gateways shall be improved through the use of appropriate signage, landscaping, by more uniform setbacks, building design, public art, and amenities. The seven gateways are: Saratoga Road (Route 50), Freemans Bridge Road (Route 911F), Amsterdam Road (Route 5), Sacandaga Road (Route 147), Glenridge Road (Route 914V), the Exit 26 bridge, and the Route 50/Village of Scotia municipal boundary.

- Any commercial establishment adjacent to, or within  1,000  feet of, a Town gateway shall contribute  to  enhancement  of the gateway through additional landscaping, as  required  by the Planning and Zoning Commission. Examples of potential contributions	include	perennial	low-maintenance groundcover, use of native trees and native plantings.

- Application procedures.  The  application  procedures  to  be followed are the same as for those required of site plan review applications as detailed in § 270-108 of the Zoning Code.